GPL951XXUA PWMIO Driver Code

User’s Manual

V0.1 – March 21, 2017

Important Notice

Generalplus Technology reserves the right to change this documentation without prior notice. Information provided by Generalplus Technology is believed to be accurate and reliable. However, Generalplus Technology makes no warranty for any errors which may appear in this document. Contact Generalplus Technology to obtain the latest version of device specifications before placing your order. No responsibility is assumed by Generalplus Technology for any infringement of patent or other rights of third parties which may result from its use. In addition, Generalplus products are not authorized for use as critical components in life support devices/ systems or aviation devices/systems, where a malfunction or failure of the product may reasonably be expected to result in significant injury to the user, without the express written approval of Generalplus.

Table of Content

PAGE

1	Introduction	6

2	Function List for Assembly	8

3	Global Variable for Assembly	9

3.1	PWMIO Driver Global Variable	9

4	Global Table for Assembly	9

4.1	PWMIO Driver Global Table	9

5	Resource	9

5.1	PWMIO Driver Code	9

5.1.1	RAM Size	9

5.1.2	ROM Size	9

6	Project Architecture for Assembly	10

6.1	Architecture	10

7	Application Interface for Assembly	11

7.1	PWMIO driver Code	11

7.1.1	F_PWMIO_Initial	11

7.1.2	F_PWMIO_Duty_Setting	11

7.1.3	F_PWMIO_Freq_Setting	12

7.1.4	F_PWMIO_Start	12

7.1.5	F_PWMIO_Stop	13

8	Function List for C	14

9	Project Architecture for C	16

9.1	Architecture	16

10	Application Interface for C	17

10.1	PWMIO Driver Code	17

10.1.1	PWMIO_Initial	17

10.1.2	PWMIO_Duty_Setting	17

10.1.3	PWMIO_Freq_Setting	18

10.1.4	PWMIO_Start	18

10.1.5	PWMIO_Stop	19

11	Program Example	19

12	Special Note	21

Revision History

| Revision | Date | By | Remark |
| --- | --- | --- | --- |
| 0.1 | 2017.03.21 | Kingchao | First Edition |

# Introduction

GPL951XX contains eight 16-bit up-count timer/counters: TimerA~TimerH. TimerA~TimerD support Pulse-Width-Modulation (PWM) function when cooperates with their own two 16-bit registers (preload registers and CCPB registers).

When timer/counter is set as in Pulse-Width-Modulation (PWM) mode, the PortB[3:0] pin is configured as OUTPUT pin automatically. When the timer/counter is enabled, the PWM output will toggle at the overflow of timer/counter and at the end of CCPB cycle. There are two output signal polarities on PWM output pin, positive and negative.

In Pulse-Width-Modulation (PWM) normal mode

PWM frequency is set by the preloaded register value. See the formula below:

PWM Output Frequency = (Source Clock Frequency) / (65536 – Preload Register Value)

PWM duty cycle is set by the CCPB register value. See the formula below:

PWM Output Duty Cycle =(CCPB Register - Preload_Register+1) / (65536 – Preload Register)

PWM period = 1 / PWM Frequency

# Function List for Assembly

| PWMIO Driver Function List |  |  |  |
| --- | --- | --- | --- |
| Function | Parameter | Return | Description |
| F_PWMIO_Initial | None | None | PWMIO driver and HW initialize |
| F_ PWMIO_Duty_Setting | R1[11:8] : PWMIO channel select 0x0100 – PWMIO 0 0x0200 – PWMIO 1 0x0400 – PWMIO 2 0x0800 – PWMIO 3 R2 : PWMIO duty setting, | None | PWMIO channel duty setting |
| F_PWMIO_Freq_Setting | R1[11:8] : PWMIO channel select 0x0100 – PWMIO 0 0x0200 – PWMIO 1 0x0400 – PWMIO 2 0x0800 – PWMIO 3 R2 : PWMIO frequency setting, | None | PWMIO channel frequency setting |
| F_PWMIO_Start | R1[11:8] : PWMIO channel select 0x0100 – PWMIO 0 0x0200 – PWMIO 1 0x0400 – PWMIO 2 0x0800 – PWMIO 3 | None | Start PWMIO signal output |
| F_PWMIO_Stop | R1[11:8] : PWMIO channel select 0x0100 – PWMIO 0 0x0200 – PWMIO 1 0x0400 – PWMIO 2 0x0800 – PWMIO 3 | None | Stop PWMIO signal output |

# Global Variable for Assembly

## PWMIO Driver Global Variable

| Name | Size(word) | Description |
| --- | --- | --- |
| None | None | None |

# Global Table for Assembly

## PWMIO Driver Global Table

| Name | Size(word) | Description |
| --- | --- | --- |
| None | None | None |

# Resource

## PWMIO Driver Code

### RAM Size

| Overlap RAM (word) | Non-overlap RAM (word) |
| --- | --- |
| 0 | 0 |

### ROM Size

| Code (word) | DATA (word) |
| --- | --- |
| 109 | 0 |

# Project Architecture for Assembly

## Architecture

Project

main.asm

PWMIO_INC.inc

PWMIO_Define.inc

isr.asm

PWMIO.asm

# Application Interface for Assembly

## PWMIO driver Code

### F_PWMIO_Initial

| Description | PWMIO HW initial |
| --- | --- |
| Parameters | None |
| Return Values | None |
| Destroy | R1 |
| Remarks | none |
| Example | …….. call F_PWMIO_Initial …….. |

### F_PWMIO_Duty_Setting

| Description | Set PWMIO duty |
| --- | --- |
| Parameters | R1[11:8] : PWMIO channel select 0x0100 – select PWMIO0 0x0200 – select PWMIO1 0x0400 – select PWMIO2 0x0800 – select PWMIO3 R2 : PWMIO duty setting |
| Return Values | None |
| Destroy | R1, R2 |
| Remarks | User need call this function to set PWMIO duty |
| Example | PWMIO_2kHz .EQU (0x10000ul-(systemclock/2/2000)) …….. r1 =PWMIO0 //set PWMIO0 1/4 duty r2 = PWMIO_2kHz+((0x10000ul-PWMIO_2kHz)/4)) call F_PWMIO_Duty_Setting r1 =PWMIO1 //set PWMIO1 1/8 duty r2 = PWMIO_2kHz+((0x10000ul-PWMIO_2kHz)/8)) call F_PWMIO_Duty_Setting …….. |

### F_PWMIO_Freq_Setting

| Description | Set PWMIO Frequency |
| --- | --- |
| Parameters | R1[11:8] : PWMIO channel select 0x0100 – select PWMIO0 0x0200 – select PWMIO1 0x0400 – select PWMIO2 0x0800 – select PWMIO3 R2 : PWMIO frequency setting |
| Return Values | None |
| Destroy | R1, R2 |
| Remarks | User need call this function to set PWMIO frequency |
| Example | PWMIO_2kHz .EQU (0x10000ul-(systemclock/2/2000)) PWMIO_2kHz .EQU (0x10000ul-(systemclock/2/4000)) …….. r1 =PWMIO0 //set PWMIO0 2kHz r2 = PWMIO_2kHz call F_PWMIO_Freq_Setting r1 =PWMIO1 //set PWMIO1 4kHz r2 = PWMIO_2kHz call F_PWMIO_Freq_Setting …….. |

### F_PWMIO_Start

| Description | Start output PWMIO signal |
| --- | --- |
| Parameters | R1[11:8] : PWMIO channel select 0x0100 – select PWMIO0 0x0200 – select PWMIO1 0x0400 – select PWMIO2 0x0800 – select PWMIO3 |
| Return Values | None |
| Destroy | R1, R2 |
| Remarks | none |
| Example | PWMIO_2kHz .EQU (0x10000ul-(systemclock/2/2000)) …….. call F_PWMIO_Initial(); //PWMIO initial r1 =PWMIO0 //set PWMIO0 2kHz r2 = PWMIO_2kHz call F_PWMIO_Freq_Setting r1 =PWMIO0 //set PWMIO0 1/4 duty r2 = PWMIO_2kHz+((0x10000ul-PWMIO_2kHz)/4)) r1 =PWMIO0 //Start PWMIO0 call F_PWMIO_Start …….. |

### F_PWMIO_Stop

| Description | Stop output PWMIO signal |
| --- | --- |
| Parameters | R1[113:8] : PWMIO channel select 0x0100 – select PWMIO0 0x0200 – select PWMIO1 0x0400 – select PWMIO2 0x0800 – select PWMIO3 |
| Return Values | None |
| Destroy | R1, R2 |
| Remarks | none |
| Example | …….. r1 =PWMIO0 //Stop PWMIO0 call F_PWMIO_Stop …….. |

# Function List for C

| PWMIO Driver Function List |  |  |  |
| --- | --- | --- | --- |
| Function | Parameter | Return | Description |
| void PWMIO_Initial(void); | None | None | PWMIO Driver and HW Initialize |
| void PWMIO_Duty_Setting(unsigned int PWMIO_Ch unsigned int PWMIO_Duty); | unsigned int PWMIO_Ch : channel select 0x0100 – PWMIO0 0x0200 – PWMIO1 0x0400 – PWMIO2 0x0800 – PWMIO3 unsigned int PWMIO_Duty : duty setting | None | PWMIO channel duty setting |
| void PWMIO_Freq_Setting(unsigned int PWMIO_Ch unsigned int PWMIO_Freq); | unsigned int PWMIO_Ch : channel select 0x0100 – PWMIO0 0x0200 – PWMIO1 0x0400 – PWMIO2 0x0800 – PWMIO3 unsigned int PWMIO_Freq : frequency setting | None | PWMIO channel frequency setting |
| void PWMIO_Start(unsigned int PWMIO_Ch); | unsigned int PWMIO_Ch : channel select 0x0100 – PWMIO0 0x0200 – PWMIO1 0x0400 – PWMIO2 0x0800 – PWMIO3 | None | Start PWMIO signal output |
| void PWMIO_Stop(unsigned int PWMIO_ch); | unsigned int PWMIO_Ch : channel select 0x0100 – PWMIO0 0x0200 – PWMIO1 0x0400 – PWMIO2 0x0800 – PWMIO3 | None | Stop PWMIO signal output |

# Project Architecture for C

## Architecture

Project

main.c

PWMIO_H.h

PWMIO_Define.inc

isr.c

PWMIO.asm

# Application Interface for C

## PWMIO Driver Code

### PWMIO_Initial

| Description | PWMIO Driver and HW initialize |
| --- | --- |
| Header File | PWMIO_H.h |
| Syntax | void PWMIO_Initial(void); |
| Parameters | None |
| Return Values | None |
| Remarks | Programmers must call this function to initial PWMIO driver before user PWMIO function |
| Example | #include PWMIO_H.h int main(void) { …….. PWMIO_Initial(); …….. } |

### PWMIO_Duty_Setting

| Description | Set PWMIO duty |
| --- | --- |
| Header File | PWMIO_H.h |
| Syntax | void PWMIO_Duty_Setting(unsigned int PWMIO_Ch, unsigned int PWMIO_Duty); |
| Parameters | PWMIO_Ch[11:8] : PWMIO channel select 0x0100 – select PWMIO0 0x0200 – select PWMIO1 0x0400 – select PWMIO2 0x0800 – select PWMIO3 PWMIO_Duty : PWMIO duty setting |
| Return Values | None |
| Remarks | User need call this function to set PWMIO duty |
| Example | #define PWMIO_2kHz (0x10000ul-(systemclock/2/2000)) #define PWMIO_Duty_4 PWMIO_2kHz+((0x10000ul-PWMIO_2kHz)/4)) #define PWMIO_4kHz (0x10000ul-(systemclock/2/4000)) #define PWMIO_Duty_8 PWMIO_4kHz+((0x10000ul-PWMIO_4kHz)/8)) …….. //set PWMIO0 1/4 duty PWMIO_Duty_Setting(PWMIO0, PWMIO_Duty_4) //set PWMIO1 1/4 duty PWMIO_Duty_Setting(PWMIO1, PWMIO_Duty_8) …….. |

### PWMIO_Freq_Setting

| Description | Set PWMIO Frequency |
| --- | --- |
| Header File | PWMIO_H.h |
| Syntax | void PWMIO_Freq_Setting(unsigned int PWMIO_Ch, unsigned int PWMIO_Freq); |
| Parameters | PWMIO_Ch[11:8] : PWMIO channel select 0x0100 – select PWMIO0 0x0200 – select PWMIO1 0x0400 – select PWMIO2 0x0800 – select PWMIO3 PWMIO_Duty : PWMIO frequency setting |
| Return Values | None |
| Remarks | User need call this function to set PWMIO frequency |
| Example | #define PWMIO_2kHz (0x10000ul-(systemclock/2/2000)) #define PWMIO_Duty_4 PWMIO_2kHz+((0x10000ul-PWMIO_2kHz)/4)) #define PWMIO_4kHz (0x10000ul-(systemclock/2/4000)) #define PWMIO_Duty_8 PWMIO_4kHz+((0x10000ul-PWMIO_4kHz)/8)) …….. //set PWMIO0 2kHz PWMIO_ Freq _Setting(PWMIO0, PWMIO_2kHz) //set PWMIO1 2kHz PWMIO_ Freq _Setting(PWMIO1, PWMIO_4kHz) …….. |

### PWMIO_Start

| Description | Start output PWMIO signal |
| --- | --- |
| Header File | PWMIO_H.h |
| Syntax | void PWMIO_Start(unsigned int PWMIO_Ch); |
| Parameters | unsigned int PWMIO_Ch PWMIO_Ch [13:8] : PWMIO channel select 0x0100 – select PWMIO0 0x0200 – select PWMIO1 0x0400 – select PWMIO2 0x0800 – select PWMIO3 |
| Return Values | None |
| Remarks | None |
| Example | #include PWMIO_H.h #define PWMIO_2kHz (0x10000ul-(systemclock/2/2000)) #define PWMIO_Duty_4 PWMIO_2kHz+((0x10000ul-PWMIO_2kHz)/4)) int main(void) { …….. PWMIO_Initial(); //set PWMIO0 2kHz, 1/4 duty PWMIO_ Freq _Setting(PWMIO0, PWMIO_2kHz) PWMIO_Duty_Setting(PWMIO0, PWMIO_Duty_4) PWMIO_Start(PWMIO0); …….. } |

### PWMIO_Stop

| Description | Stop output PWMIO signal |
| --- | --- |
| Header File | PWMIO_H.h |
| Syntax | void PWMIO_Stop(unsigned int PWMIO_Ch); |
| Parameters | unsigned int PWMIO_Ch PWMIO_Ch [13:8] : PWMIO channel select 0x0100 – select PWMIO0 0x0200 – select PWMIO1 0x0400 – select PWMIO2 0x0800 – select PWMIO3 |
| Return Values | None |
| Remarks | None |
| Example | #include PWMIO_H.h int main(void) { …….. PWMIO_Initial(); //set PWMIO0 2kHz, 1/4 duty PWMIO_ Freq _Setting(PWMIO0, PWMIO_2kHz) PWMIO_Duty_Setting(PWMIO0, PWMIO_Duty_4) PWMIO_Start(PWMIO0); …….. …….. PWMIO_Stop(PWMIO0); // PWMIO0 stop } |

# Program Example

In maic.c

//main.c

#include "GPL951_Body.h"

#include "PWMIO_H.h"

#define PWMIO_2kHz	(0x10000ul-(systemclock/2/2000))

;

int main()

{

//add your code here

*P_IOA_Dir = 0x00;

*P_IOA_Attrib = 0x00;

*P_IOA_Buffer = 0x00;

*P_IOA_Data = 0x00;

PWMIO_Initial();

//PWMIO0/2 set 2kHz, 1/4 duty

PWMIO_Freq_Setting(PWMIO0+PWMIO1+PWMIO2+PWMIO3, PWMIO_2kHz);

PWMIO_Duty_Setting(PWMIO0+PWMIO2, (PWMIO_2kHz+((0x10000ul-PWMIO_2kHz)/4)));			//PWMIO1/3 set 2kHz, 1/8 duty

PWMIO_Duty_Setting(PWMIO1+PWMIO3, (PWMIO_2kHz+((0x10000ul-PWMIO_2kHz)/8)));

PWMIO_Start(PWMIO0+PWMIO1+PWMIO2+PWMIO3);

while(*P_IOA_Data==0)

{

*P_WatchDog_Clear = C_WDT_Clear;		//clear WDT

}

PWMIO_Stop(PWMIO1+PWMIO3);

while(1)

{

*P_WatchDog_Clear = C_WDT_Clear;		//clear WDT

}

return 0;

}

# Special Note

- In PWMIO_Define.inc, it has some constant define. Programmers need to define these parameters before use PWMIO driver.

//==========================================

// Constant define area

//==========================================

.define PWMIO0 1				//1: Enable IOB0 as PWMIO, refer to TimerA setting

.define PWMIO0_Polarity	0		//0: Output low pulse

//1: Output high pulse

.define PWMIO1 0				//1: Enable IOB1 as PWMIO, refer to TimerB setting

.define PWMIO1_Polarity	0		//0: Output low pulse

//1: Output high pulse

.define PWMIO2 0				//1: Enable IOB2 as PWMIO, refer to TimerC setting

.define PWMIO2_Polarity	0		//0: Output low pulse

//1: Output high pulse

.define PWMIO3 0				//1: Enable IOB3 as PWMIO, refer to TimerD setting

.define PWMIO3_Polarity	0		//0: Output low pulse

//1: Output high pulse