GPL951XXUA BAM Driver Code

User’s Manual

V0.1 – March 21, 2017

Important Notice

Generalplus Technology reserves the right to change this documentation without prior notice. Information provided by Generalplus Technology is believed to be accurate and reliable. However, Generalplus Technology makes no warranty for any errors which may appear in this document. Contact Generalplus Technology to obtain the latest version of device specifications before placing your order. No responsibility is assumed by Generalplus Technology for any infringement of patent or other rights of third parties which may result from its use. In addition, Generalplus products are not authorized for use as critical components in life support devices/ systems or aviation devices/systems, where a malfunction or failure of the product may reasonably be expected to result in significant injury to the user, without the express written approval of Generalplus.

Table of Content

PAGE

1	Introduction	6

2	Function List for Assembly	8

3	Global Variable for Assembly	9

3.1	BAM Driver Global Variable	9

4	Global Table for Assembly	9

4.1	BAM Driver Global Table	9

5	Resource	9

5.1	BAM Driver Code	9

5.1.1	RAM Size	9

5.1.2	ROM Size	9

6	Project Architecture for Assembly	10

6.1	Architecture	10

7	Application Interface for Assembly	11

7.1	BAM driver Code	11

7.1.1	F_BAM_Initial	11

7.1.2	F_BAM_Data_Setting	11

7.1.3	F_BAM_Start	12

7.1.4	F_BAM_Stop	12

8	Function List for C	13

9	Project Architecture for C	15

9.1	Architecture	15

10	Application Interface for C	16

10.1	BAM Driver Code	16

10.1.1	BAM_Initial	16

10.1.2	BAM_Data_Setting	16

10.1.3	BAM_Start	16

10.1.4	BAM_Stop	17

11	Program Example	18

12	Special Note	19

Revision History

| Revision | Date | By | Remark |
| --- | --- | --- | --- |
| 0.1 | 2017.03.21 | Kingchao | First Edition |

# Introduction

GPL951XX contains eight 16-bit up-count timer/counters: TimerA~TimerH. TimerA~TimerF support Bit-Angle-Modulation (BAM) function when cooperates with their own two 16-bit registers (preload registers and CCPB registers).

When timer/counter is set as in Bit-Angle-Modulation (BAM) mode, the PortB[5:0] pin is configured as OUTPUT pin automatically. When the timer/counter is enabled, the BAM output will toggle at the overflow of timer/counter and at the end of CCPB cycle. There are two output signal polarities on BAM output pin, positive and negative.

In Bit-Angle-Modulation(BAM) Mode

# Function List for Assembly

| BAM Driver Function List |  |  |  |
| --- | --- | --- | --- |
| Function | Parameter | Return | Description |
| F_BAM_Initial | None | None | BAM driver and HW initialize |
| F_BAM_Data_Setting | R1[13:8] : BAM channel select 0x0100 – BAM0 0x0200 – BAM1 0x0400 – BAM2 0x0800 – BAM3 0x1000 – BAM4 0x2000 – BAM5 R1[7:0] : BAM data, 0x00~0xff | None | BAM channel data setting |
| F_BAM_Start | R1[13:8] : BAM channel select 0x0100 – BAM0 0x0200 – BAM1 0x0400 – BAM2 0x0800 – BAM3 0x1000 – BAM4 0x2000 – BAM5 | None | Start BAM signal output |
| F_BAM_Stop | R1[13:8] : BAM channel select 0x0100 – BAM0 0x0200 – BAM1 0x0400 – BAM2 0x0800 – BAM3 0x1000 – BAM4 0x2000 – BAM5 | None | Stop BAM signal output |

# Global Variable for Assembly

## BAM Driver Global Variable

| Name | Size(word) | Description |
| --- | --- | --- |
| None | None | None |

# Global Table for Assembly

## BAM Driver Global Table

| Name | Size(word) | Description |
| --- | --- | --- |
| None | None | None |

# Resource

## BAM Driver Code

### RAM Size

| Overlap RAM (word) | Non-overlap RAM (word) |
| --- | --- |
| 0 | 0 |

### ROM Size

| Code (word) | DATA (word) |
| --- | --- |
| 198(1ch) ~ 284(6ch) | 0 |

# Project Architecture for Assembly

## Architecture

Project

main.asm

BAM_INC.inc

BAM_Define.inc

isr.asm

BAM.asm

# Application Interface for Assembly

## BAM driver Code

### F_BAM_Initial

| Description | BAM HW initial |
| --- | --- |
| Parameters | None |
| Return Values | None |
| Destroy | R1 |
| Remarks | none |
| Example | …….. call F_BAM_Initial …….. |

### F_BAM_Data_Setting

| Description | Set BAM data |
| --- | --- |
| Parameters | R1[13:8] : BAM channel select 0x0100 – select BAM0 0x0200 – select BAM1 0x0400 – select BAM2 0x0800 – select BAM3 0x1000 – select BAM4 0x2000 – select BAM5 R1[7:0] : BAM data, 0x00~0xff |
| Return Values | None |
| Destroy | R1, R2 |
| Remarks | User need call this function to set BAM data |
| Example | …….. r1 = BAM0+0xff //set BAM0 data 0xff call F_BAM_Data_Setting r1 = BAM3+0x80 //set BAM3 data 0x80 call F_BAM_Data_Setting r1 = BAM1 + BAM5+0x30 //set BAM1 and BAM5 data 0x30 call F_BAM_Data_Setting …….. |

### F_BAM_Start

| Description | Start output BAM signal |
| --- | --- |
| Parameters | R1[13:8] : BAM channel select 0x0100 – select BAM0 0x0200 – select BAM1 0x0400 – select BAM2 0x0800 – select BAM3 0x1000 – select BAM4 0x2000 – select BAM5 |
| Return Values | None |
| Destroy | R1, R2 |
| Remarks | none |
| Example | …….. call F_BAM_Initial(); //BAM initial r1 = BAM0 + 0x0080; call F_BAM_Data_Setting; //BAM0 data 0x0080 r1 = BAM0; call F_BAM_Start; //Start output BAM0 signal …….. |

### F_BAM_Stop

| Description | Stop output BAM signal |
| --- | --- |
| Parameters | R1[13:8] : BAM channel select 0x0100 – select BAM0 0x0200 – select BAM1 0x0400 – select BAM2 0x0800 – select BAM3 0x1000 – select BAM4 0x2000 – select BAM5 |
| Return Values | None |
| Destroy | R1, R2 |
| Remarks | none |
| Example | …….. r1 = BAM0; call F_BAM_Stop; //Stop BAM0 …….. |

# Function List for C

| BAM Driver Function List |  |  |  |
| --- | --- | --- | --- |
| Function | Parameter | Return | Description |
| void BAM_Initial(void); | None | None | BAM Driver and HW Initialize |
| void BAM_Data_Setting(unsigned int setting); | Unsigned int setting setting[13:8] : BAM channel select 0x0100 – BAM0 0x0200 – BAM1 0x0400 – BAM2 0x0800 – BAM3 0x1000 – BAM4 0x2000 – BAM5 setting[7:0] : BAM data, 0x00~0xff | None | BAM channel data setting |
| void BAM_Start(unsigned int BAM_ch); | Unsigned int BAM_ch BAM_ch[13:8] : BAM channel select 0x0100 – BAM0 0x0200 – BAM1 0x0400 – BAM2 0x0800 – BAM3 0x1000 – BAM4 0x2000 – BAM5 | None | Start BAM signal output |
| void BAM_Stop(unsigned int BAM_ch); | Unsigned int BAM_ch BAM_ch[13:8] : BAM channel select 0x0100 – BAM0 0x0200 – BAM1 0x0400 – BAM2 0x0800 – BAM3 0x1000 – BAM4 0x2000 – BAM5 | None | Stop BAM signal output |

# Project Architecture for C

## Architecture

Project

main.c

BAM_H.h

BAM_Define.inc

isr.c

BAM.asm

# Application Interface for C

## BAM Driver Code

### BAM_Initial

| Description | BAM Driver and HW initialize |
| --- | --- |
| Header File | BAM_H.h |
| Syntax | void BAM_Initial(void); |
| Parameters | None |
| Return Values | None |
| Remarks | Programmers must call this function to initial BAM driver before user BAM function |
| Example | #include BAM_H.h int main(void) { …….. BAM_Initial(); …….. } |

### BAM_Data_Setting

| Description | Set BAM data |
| --- | --- |
| Header File | BAM_H.h |
| Syntax | void BAM_Data_Setting(unsigned int setting); |
| Parameters | setting[13:8] : BAM channel select 0x0100 – select BAM0 0x0200 – select BAM1 0x0400 – select BAM2 0x0800 – select BAM3 0x1000 – select BAM4 0x2000 – select BAM5 setting[7:0] : BAM data, 0x00~0xff |
| Return Values | None |
| Remarks | User need call this function to set BAM data |
| Example | #include BAM_H.h int main(void) { …….. BAM_Initial(); BAM_data_Setting(BAM0+BAM3+0x00ff); //BAM0 and BAM3 data 0x00ff …….. } |

### BAM_Start

| Description | Start output BAM signal |
| --- | --- |
| Header File | BAM_H.h |
| Syntax | void BAM_Start(unsigned int BAM_ch); |
| Parameters | unsigned int BAM_ch BAM_ch [13:8] : BAM channel select 0x0100 – select BAM0 0x0200 – select BAM1 0x0400 – select BAM2 0x0800 – select BAM3 0x1000 – select BAM4 0x2000 – select BAM5 |
| Return Values | None |
| Remarks | None |
| Example | #include BAM_H.h int main(void) { …….. BAM_Initial(); BAM_data_Setting(BAM0+BAM3+0x00ff); //BAM0 and BAM3 data 0x00ff BAM_Start(BAM0+BAM3); //BAM0 and BAM3 start …….. } |

### BAM_Stop

| Description | Stop output BAM signal |
| --- | --- |
| Header File | BAM_H.h |
| Syntax | void BAM_Stop(unsigned int BAM_ch); |
| Parameters | unsigned int BAM_ch BAM_ch [13:8] : BAM channel select 0x0100 – select BAM0 0x0200 – select BAM1 0x0400 – select BAM2 0x0800 – select BAM3 0x1000 – select BAM4 0x2000 – select BAM5 |
| Return Values | None |
| Remarks | None |
| Example | #include BAM_H.h int main(void) { …….. BAM_Initial(); BAM_data_Setting(BAM0+BAM3+0x00ff); //BAM0 and BAM3 data 0x00ff BAM_Start(BAM0+BAM3); //BAM0 and BAM3 start …….. BAM_Stop(BAM3); // BAM3 stop } |

# Program Example

In maic.c

//main.c

#include "GPL951_Body.h"

#include "BAM_H.h"

int main()

{

unsigned int BAM_Data;

//add your code here

*P_IOC_Dir = 0x00;

*P_IOC_Attrib = 0x00;

*P_IOC_Buffer = 0x00;

*P_IOC_Data = 0x00;

BAM_Initial();			//BAM initial

BAM_Data = *P_IOC_Data&0x00ff;							//get IOC[7:0] as BAM data

BAM_Data_Setting(BAM0+BAM2+BAM4+BAM_Data);			//BAM0/2/4 output BAM_Data

BAM_Data_Setting(BAM1+BAM3+BAM5+(BAM_Data^0xff));		//BAM1/3/5 output BAM_Data^0xff

BAM_Start(BAM0+BAM1+BAM2+BAM3+BAM4+BAM5);			//Start output BAM signal

while(1)

{

BAM_Data = *P_IOC_Data&0x00ff;						//get IOC[7:0] as BAM data

BAM_Data_Setting(BAM0+BAM2+BAM4+BAM_Data);		//BAM0/2/4 output BAM_Data

BAM_Data_Setting(BAM1+BAM3+BAM5+(BAM_Data^0xff));	//BAM1/3/5 output BAM_Data^0xff

*P_WatchDog_Clear = C_WDT_Clear;					//clear WDT

}

return 0;

}

# Special Note

- In BAM_Define.inc, it has some constant define. Programmers need to define these parameters before use BAM driver.

//==========================================

// Constant define area

//==========================================

.define BAM0 1				//1: Enable IOB0 as BAM, refer to TimerA setting

.define BAM0_Polarity	0		//0: Output low pulse

//1: Output high pulse

.define BAM1 0				//1: Enable IOB1 as BAM, refer to TimerB setting

.define BAM1_Polarity	0		//0: Output low pulse

//1: Output high pulse

.define BAM2 0				//1: Enable IOB2 as BAM, refer to TimerC setting

.define BAM2_Polarity	0		//0: Output low pulse

//1: Output high pulse

.define BAM3 0				//1: Enable IOB3 as BAM, refer to TimerD setting

.define BAM3_Polarity	0		//0: Output low pulse

//1: Output high pulse

.define BAM4 0				//1: Enable IOB4 as BAM, refer to TimerE setting

.define BAM4_Polarity	0		//0: Output low pulse

//1: Output high pulse

.define BAM5 0				//1: Enable IOB5 as BAM, refer to TimerF setting

.define BAM5_Polarity	0		//0: Output low pulse

//1: Output high pulse